CS 250 Midterm 2

Chapter 7:

- 0-address (stack architecture) kept on run-time stack. (page. 142)

- 1-address limits each instruction to a single operand (page. 142)

- 2-address, an operation can be applied to a specified value instead of merely to the accumulator. (page. 143)

- 3-address, two input values and destination (page. 144)

- An anti-dependence between J and K occurs when K writes a register or memory location that J reads and instruction J is executed first

- An output dependence occurs when J and K write the same register or memory location

- Pipelining does not improve execution time of any single instruction

- Each instruction takes longer to execute than in a single-cycle datapath because of